Python-based numerical modeling created creep rupture curves and estimated residual lifetime.

import numpy as np

import matplotlib.pyplot as plt

Operation _Temperature in Celsius = 800.0

My stress Inventor Von Mises = 14.57 (MPa)

C Stainless Steel = 20.0

Temperature in _kelvin = operation _temperature _C + 273.15

stress points = np.array([5.0, 10.0, 20.0, 30.0, 50.0, 100.0, 150.0, 200.0])

#Values of Lardon-Miller Parameter multiplied by 3^10

lmp_points = np. array ([25.8, 24.5, 23.2, 22.4, 21.4, 20.0, 19.2, 18.5]) * 1000

#To create a smooth and continuous curve (interpolation)

stress range = np.linspace(4.0, 200.0, 500)

lmp_range = np.interp(stress range, stress points, lmp_points)

# log10(t) = (LMP / T) - C

Log_time = (lmp_range / T_kelvin) - C

Time_to_rupture = 10**log_time

#Calculation of the precise Residual time for the stress value for your design (14.57 MPa)

My_lmp = np. interp (my_stress, stress points, lmp_points)

My_log_time = (my_lmp / T_kelvin) - C

My_time_to_rupture = 10**my_log_time

#Printing the digital result in the outputs screen

print ("-" * 50)

print (f"--- The results of predicting the residual age for the combustion chamber ---")

print (f"the designed Von Mises Stress: {my_stress} MPa")

print (operation Temperature: {operation_temp_C} °C")

print (f" Rupture Prediction Life time: {my_time_to_rupture:,.1f} ")

print ("-" * 50)

(Stress vs. Time to Rupture)

plt. Figure (figsize= (10, 6))

#Drawing the basic curve for the material

plt. Plot (time_to_rupture, stress range, '-', color='#1f77b4', linewidth=2.5, label=f'SS316/304 Rupture Curve @ {operation _temp_C}°C')

#Placing your design point on the curve

plt.plot(my_time_to_rupture, my_stress, 'ro', markersize=8, label=f'Your Design ({my_stress} MPa)')

#Drawing crossed instructing lines for the design point

plt.axhline(y=my_stress, color='r', linestyle=':', alpha=0.6)

plt.axvline(x=my_time_to_rupture, color='r', linestyle=':', alpha=0.6)

plt.text(my_time_to_rupture * 1.5, my_stress * 1.2,

f"RUL ~ {my_time_to_rupture:,.0f} Hours\n(~ {my_time_to_rupture/(24*365):.1f} Years)",

color='black', weight='bold', bbox=dict(facecolor='yellow', alpha=0.3))

#Coordinating the drawing axis and the log. settings

plt.xscale('log')

plt.grid(True, which="both", linestyle="--", alpha=0.5)

plt.title('Creep Life Prediction using Larson-Miller Parameter (LMP)', fontsize=12, weight='bold')

plt.xlabel('Time to Rupture, $t_r$ (Hours) - Log Scale', fontsize=11)

plt.ylabel('Von Mises Stress, $\sigma_v$ (MPa)', fontsize=11)

plt.xlim(10, 1e6) # the zone show from 10 hours to million hours

plt.ylim(0, 120)

plt.legend(loc='upper right')

# the diagram show

plt.tight_layout()

plt.show()